Power Removed Control Product Category Reference Sheet

BYPASS MAP:  For areas where the Bypass Map is in control, these are the colors that are used. UNFILTERED means there was no clutter filter power removed for that bin.  NO CLUTTER means there was clutter filter power removed, but it was less than 0.5 dB.  LOW/MED/HIGH show bins were clutter filter power was removed in ranges of < 25 dB, 25-50 dB, or > 50 dB.

NO FILTER:  For areas where no filtering should be occurring either because of a forced no filter clutter zone (FORCE NO FILTER) or because there was an error and a forced no filter clutter zone was in place but filtering occurred in error (FILTER ERROR).

ALL BINS:  For areas where all-bins clutter filtering is in control, these are the colors that are used.  NO CLUTTER indicates where filtering was applied but it was less than 0.5 dB.  LOW/MED/HIGH shows bins where clutter filtering was forced by all bins and the power removed was < 25 dB, 25-50 dB, or > 50 dB.

POINT CLUTTER:  Indicates if an individual bin has power return from a clutter target that isn’t necessarily distributed in nature (e.g., aircraft).

DP FILTERED:  A RhoHV test is applied to help clean up the dual-pol variables in the typical clutter area.  Bins where the RhoHV test modifies the dual-pol variables will have this bin applied to it in the PRC product.  *NOTE:  DP FILTERED only means dual-pol variables were filtered.  No Z was filtered.*